This file is basically the text of the on-line help.

LA General Information

The on-line help is not a substitute for reading the documentation set distributed with the software.  Most of the operational information is replicated here.  There is a good possibility that I have forgotten to include some details, however this is my first attemp at creating a help facility and hope that you find it useful.

The following files MUST be in folder c:\LA_32

InpOut32.exe

LA.ini		;see LA.INI Sections: LA_MISC

LA_Help.hlp

LA_Help.fts

LA_Help.gid

LA32.dll

LA32.lib

I recommend that LogicAna.exe also be in c:\LA_32, but it is not necessary.

LA Versions

New features are indicated with the following notation: (VX.Y)

2.0	Upgrade to 32 bit software

use default folder for ini file

allow user to specify the default folder

added Help-Help to the menu bar

1.0	Initial release

No Hardware

The Logic Analyzer software does allow operation with no external hardware.  All it takes is for you to connect the data pins of the ECP to the signals which you want to monitor.  Although not strictly necessary, I would STRONGLY suggest that you at least buffer the signals with a 4050 in order to prevent damage to the ECP hardware.

The following feature is availabe only if the your program has been registered.

The program allows you to "calibrate" the sample period.  Please keep in mind that this is an approximate calibration.  The sample periods still must be defined in the INI file.  One of the "advantages" of this feature is that you can use unusual sample periods - values which you not normally find available if external hardware is being used.  For instance you could define the following:

[LA_TIMEBASE]

SELECT_0 = 1ms

SELECT_1 = 2ms

SELECT_2 = 3ms

SELECT_3 = 17ms

SELECT_4 = 98ms

I think you get the idea.

Unregistered Usage

If your program is not registered you have the following limitations:

you are forced into the NO_HARDWARE mode -

you cannot connect external hardware

maximum of 2 channels

cannot calibrate the sample time

you will be shown the registration screen each time you start the program

the cursor memories will not display real time values

LA.INI Sections

General Information

The INI file allows the user to pre-define many of the operating conditions for the Logic Analyzer.  Most of the controls which are on the control panel and configuration screens are available to be pre-set in this file.

Comments are allowed anywhere in the file.  Comments start with a semi-colon and continue to the end of the line.

There are several sections, similar to a standard Windows INI file, which are used to define the operation of the Logic Analyzer program.  Each section begins with its name within square brakets. For example:

[PRINTER_PORT]

The various "variables" are defined by use of the equals sign:

ADDRESS = &H278	; note that Hex notation is allowed

Any uniquenesses are discussed in the section definitions.

PRINTER_PORT

These two values MUST be defined in order to run the Logic Analyzer program:

Define the address of the EPP.  The address value may be entered in hex or decimal.  A decimal value will not have any prefix characters.

ADDRESS = &H278

Tell the program whether or not the LA hardware is being used: 0=no,  1=yes

HARDWARE = {  0, 1 }

LA_MEMORY

These two values MUST be defined in order to run the Logic Analyzer program:

Define the number of address bits used by the attached hardware.  The initial hardware design uses 17 bits.

RAM_ADDR_BITS = 17

Define the number of pre-trigger address bits . The initial  hardware design uses 12 bits.

PRE_TRIG_BITS = 12

LA_TIMEBASE

Define the sample time values.  If the hardware is being used the user must define the sample time for each of the possible time bases.  The syntax of the variables is such that it defines the value which is programmed to the hardware in order to select the defined sample time.

If there is no hardware being used the program has the ability to calibrate the sample time.  This calibration is approximate only and must be initiated by the user via the Configure menu.

The time base may be selected via the "Configure" menu.

The general syntax of the variable definition is:

SELECT_A = BBBcc

where:

A is the value to program to the hardware.  It is also the number which is used to select

which sample time to use.  The range of values is: 0 <= A <= 31

BBB is the numeric portion of the sample time.  Any "reasonable" positive integer is valid.

cc is the suffix.  The allowable suffixes are: ns, us, and ms: nano-seconds, micro-seconds,

and milli-seconds respectively.   For example:

SELECT_3 = 10us

SELECT_4 = 100us

SELECT_5 = 1ms

Another variable which may be defined in this section allows the user to pre-define which sample time to use as the default when the program is started:

VALUE = 4		;select 100us

LA_CHANNELS

This section allows the user to define the number of channels to be used as well a name for each of the channels.  The names must be less than 13 characters each.  Any names exceeding 13 characters will be truncated.

These values may be defined via the "Configure" menu.

The general syntax for naming a channel is:

NAME_CHx = Name

To define the number of channels:

COUNT = x

where x represents the channel number: 1 <= x <= 8

If you do not define the number of channels the program will use 2.

You may also define the line weight and color of the logic levels - these are the defaults:

COLOR_0 = 0		;black

COLOR_1 = 4		;bright red

WEIGHT_0 = 4		;fairly heavy line

WEIGHT_1 = 2		;medium weight

LA_TRIGGER

This section lets the user pre-define the input signal conditions which will trigger the hardware.

These values may be defined via the "Configure" menu.

The general syntax is:

TRIG_CHx = { 1, 0, X }		x is the channel number

1 = trigger if the channel is a logic 1

0 = trigger if the channel is a logic 0

X = don't care

PLEASE NOTE THAT THE TRIGGERING CAPABILITY IS A VERY SIMPLE ONE.  ALL CONDITIONS MUST BE MET IN ORDER TO TRIGGER THE HARDWARE.

When the "Trigger" control is activated the hardware will begin sampling and storing the incoming data into the Pre-Trigger memory.  When the trigger conditions are met the hardware begins storing the data into the remaining memory.  When the memory is full the system will transfer the data to the LA program and display it.

If there is no hardware the system is triggered when the user presses the "Trigger" control.  When the program has collected the number of samples based on the RAM_ADDR_BITS definition it will stop sampling and then display the data.

LA_WINDOW

These variables allow the user to pre-define several start-up values for the LA.  In general these values will be modified by the user during the analysis of the data.

The following two values may be modified from the LA front panel.

START_SAMPLE = XXXX	;the starting sample number for the data window

END_SAMPLE = YYYY		;the ending sample number for the data window

Both XXXX and YYYY must fall within the range: 0 <= value < 2^RAM_ADDR_BITS.

Also the  size of the window, determined by END_SAMPLE - START_SAMPLE, is

restricted to the range: 50 <= value <= 30000.

The following two values may be defined via the "Configure" menu.

The percent by which the window will "slide" when either of the "Slide" controls is activated.  The valid range of values is:  10 <= value <= 100

SLIDE = ZZ

The value by which the window size will be multiplied wheneither of the "Zoom" controls is activated.  The valid  range of  values is:  1 <= value <= 10

ZOOM_FACTOR = ZZ

LA_CURSOR

This section allows the user to pre-define several parameters associated with the cursor.

The following two values may be modified from the LA front panel.

The initial position of the cursor:

POSITION = YYY

The initial cursor memory values:

MEM_X = YYY

X = the cursor memory number: 1 <= X <= 4

YYY = the sample number to be assigned to the cursor memory.  This must be an integer

value within the memory defined by RAM_ADDR_BITS.

The following two values may be defined via the "Configure" menu.

These variables have to do with the value displayed which represents the state of the channels at the cursor position:

SAMPLE_MSB = { OFF, CH1, CH8 }

OFF = do not display the value at the cursor

CH8 = channel 8 is the MSB

CH1 = channel 1 is the MSB

Define which number base to use in displaying the value

SAMPLE_BASE = { DEC, HEX, OCT, BIN }

LA_MEAS (V2.0)

You may pre-define the measurement conditions.  You will still need to go to the Measurement screen in order to activate the measurement.  I have shown the default values in bold type.  See Menu Bar: Measure for details on the measurement capability.

DATA_BITS = { 7, 8, 9 }

STOP_BITS = { 1, 1.5, 2 }

PARITY = { NONE, ODD, EVEN, ONE, ZERO }

MARK_STATE = { 0, 1 } This will depend on your particular implementation.

MSB_FIRST = { TRUE, FALSE } Typically in asychronous (UART) communications, the least significant bit is transmitted first.

BAUD_RATE = { enter this value in Hz } (9600)

START_CURSOR = { 1..4 } The start cursor you select must precede the start bit.

CHANNEL = { 1..8 }

DISPLAY = { ASCII, HEX, DECIMAL, OCTAL, BINARY, +TRANS, -TRANS }  Select the display mode.

LA_MISC (V2.0)

Stuff goes here that I cannot logically put somewhere else.

The user may also specify the user’s name which will be displayed on the title bar:

REG_NAME = Your Name or call

A maximum of 20 characters is allowed.

User may specify the default folder for both the data and ini files.  Upon starting LA, if this is the only statement in the file, the program will read the LA.INI file in the specified directory.

FOLDER = c:\la_data

Operation of the Logic Analyzer front panel controls

CONTROL PANEL

Most of the Control Panel settings can be set from the initialization file.  There is a hot link to the appropriate section of the LA.INI section description.  Those commands which have an associated shortcut key are indicated as such:

Draw		F2

Trigger		F3

Stop		F4

Slide Down	F5

Slide Up	F6

Zoom In	F7

Zoom Out	F8

Starting Sample Number

Set the position of the desired sample number for the left hand edge of the display window. The value may be set by either using the scroll bar controls or by typing a value into the control box.  The background color of the box will change until the "Enter" key is pressed as a visual reminder that the value has not yet been processed.

See_5 LA_WINDOWIDH_5310 for pre-defining the window starting sample.

Ending Sample Number

Set the position of the desired sample number for the right hand edge of the display window. The value may be set by either using the scroll bar controls or by typing a value into the control box.  The background color of the box will change until the "Enter" key is pressed as a visual reminder that the value has not yet been processed.

See_5 LA_WINDOWIDH_5310 for pre-defining the window ending sample.

Note: the number of samples viewed at any one time is limited to the range 50 <= number of samples <= 30000

Cursor

Set the position of the desired sample number for the cursor. The value may be set by either using the scroll bar controls or by typing a value into the control box.  The background color of the box will change until the "Enter" key is pressed as a visual reminder that the value has not yet been processed.  The cursor may also be placed by pressing the left mouse button when the pointer is over the desired location within the window.

See LA_CURSORIDH_5810 for pre-defing the cursor position.

Draw - F2

This control will draw the acquired data.  Under most circumstances the program knows when to draw the waveforms.  However, the waveforms will develop "holes" when the cursor is moved.  This control is then used to re-draw the waveforms.  I do not currently know of any method within Visual Basic which will allow the cursor to move within the display window without leaving these "holes".

Note: the time to display a signal trace is directly proportional to the number of samples to be displayed and the number of signal transitions on the channel.  It can take several seconds to draw a signal if there are 30000 samples and many transitions.  This is processor dependant - I currently have a 166MHz '686.

Trigger - F3

Activate this control in order to acquire another set of data.  While in the process of acquiring new data the status box will display the status of the acquisition:

Waiting for trigger

Triggered

Transferring

See LA_TRIGGERIDH_5210 for pre-defing the trigger parameters.

Stop - F4

This command may be used to terminate either the Draw or Trigger commands.  It is disabled unless either of the other commands has been executed and has not yet finished.  It may be necessary or desireable to sometimes terminate the Draw or Trigger commands before they would normally finish on their own.  The Draw if you are drawing a large number of samples and do not want to wait for the program to draw all the waveforms.  The Trigger if a trigger never seems to occur.

Cursor Memory Check Boxes

The Logic Analyzer allows the user to save up to 4 cursor locations.  These may be used as markers or starting point references for the measurement capability.  Each cursor memory location is indicated in the window area by a verticle line.  Each also has a different color:

1 = brown,  2 = red,  3 = orange,  4 = yellow.

A memory is activated by checking the appropriate box.  At that time the current cursor position will be stored into the memory.  There are two values displayed for each memory:

1) the cursor location, in number of samples,  when the box was checked

2) the distance, in time units, from the current cursor location

To clear a memory just "un-check" the appropriate box.

To reset a memory to another cursor location: "un-check" then "check" the box.

See LA_CURSORIDH_5210 for pre-defining the cursor memories.

Slide Up/Down - F5 / F6

These controls allow the user to "slide" the viewed data either up or down.  The window may also be caused to slide by "grabbing" the cursor with the left mouse button and dragging it to the desired position.  The data will not re-draw until the mouse button is released.

See LA_WINDOWIDH_5310 for pre-defining the slide percentage.

Zoom In/Out - F7 / F8

These two controls will allow the user to zoom in for a closer look at the data or zoom out for a more "bird's eye" view of the data.  For either command the cursor will be placed in the center of the window.

The zoom factor is set in the "Configure" menu.

See LA_WINDOW for pre-defining the zoom factor.

MENU BAR

File

Save INI

Save the current configuration into the specified .ini file.

Load INI

Configure the system based on the user specified .ini file.

Save Data

Save the current data to the specified file.  The data is stored in a text file in .CSV format.

Load Data

Load data from the specified file.  The data must have been previously stored by the program into a .CSV formatted file.

Configure

General Information

Most of the Configure settings can be set from the initialization file.  You may override the initialization file settings from the Configure panel.

There is a hot link to the appropriate section of the LA.INI section description.

Number of Active Channels

This scroll control allows the user to select the desired number of active channels.  The system always selects channels starting with channel 1.  Currently you are not allowed to select random channel number for display.

The default number of channels may be pre-selected within the INI file.

See LA_CHANNELSIDH_5110.

Channel Names

You may enter a name for each channel here.  The channel names will be printed on the main panel immediately to the left of the display for the channel.  The channel number is pre-pended to the channel name.

See LA_CHANNELSIDH_5110.

Channel Trigger State

The state of each signal to be used to trigger the Logic Analyzer.  The signals create an AND condition.  That is, each signal must be in the selected state in order for the system to trigger.  There are three choices for each of the 8 inputs:

See LA_TRIGGER.

Sample Period Selection

You may choose the sample period with this slide control.  The text printed above the bar is that which has been read from the INI file.

See LA_TIMEBASEIDH_5010.

Slide Percentage

The background color of the box will change until the "Enter" key is pressed as a visual reminder that the value has not yet been processed.

See LA_WINDOWIDH_5310.

Zoom Factor

The background color of the box will change until the "Enter" key is pressed as a visual reminder that the value has not yet been processed.

See LA_WINDOWIDH_5310.

Cursor Value Display

These allow you to control the how the value showing the state of the input channels at the cursor position is displayed.  There are 2 parts of the selection:

1) select the Most Significant Bit

a) channel 1

b) channel 8

c) no display

2) select the number base of the display

a) Decimal

b) Hex

c) Octal

d) Binary

See LA_CURSOR

Dummy Data ValuesIDH_5810

This allows the user to fill the data array with dummy data values.  This feature exists mainly to help me in debugging my drawing algorithms.  However, it can also be used as tool to help the user learn how to manipulate the front panel controls without having to collect any real data.  Tha available selections are:

1) Binary - this produces a binary sequence where the channels replicate

the patterns which would be generated by an 8 bit binary counter.

2) Pseudo Random - this will produce the same random sequence each

time you choose it.

3) Random - this will produce a different random sequence each time.

4) Zero - fills the array with zeros.

Hardware Presence Switch

This allows the user to enable or disable collecting data from the attached hardware.  Trying to enable hardware here for an unregistered version of the software will not work!

See PRINTER_PORTIDH_5610.

Calibration for No-Hardware

This allows the user to approximately calibrate the sample time.  If you want to use this feature you must manually select and then calibrate each sample time.  The values are not saved anywhere - you must calibrate each time you run the program.

Default Folder

This allows the user to select the default folder for both the data and ini files.

See LA_MISC: FOLDER

Debug

This facility is useful for debugging your hardware.  This is the software I used for ALL my hardware debugging.  I HIGHLY recommend that an oscilloscope be available to aid in building and debugging the hardware.  I believe that it will be virtually impossible to build the hardware I designed without one.

The features available in this menu are:

1) Port I/O - allows you to directly manipulate the ECP registers

a) Base Address - enter the base address of the ECP

b) Data Port - write a value to the data port.  To send the data youmust either press

the "Enter" key or click on the "Data Port" label.

c) Status Port - displays the value, in Hex, of the status port.  To display the value you

must click on the "Status Port" label.

d) Control Port - write a value to the control port.  To send the data you must either

press the "Enter" key or click on the "Control Port" label

2) Registers - allows you to interface with the registers designed into the hardware.  Most

of the operation is obvious if you are familiar with the hardware and the naming

conventions which I used.  Some useful information:

1) entering values for WP 1, 2, or 3: you must press "Enter" in order to send the value

to the hardware.

2) to read a register you must click the label above its display box.

3) the "Strobe Pulses" box allows you to send that number of strobe pulses.

4) the "Pre Trigger cycle" does the following:

a) enables sampling

b) waits for the Triggered-L signal

c) disable sampling and reset the address counters

d) optionally write the pre-trigger data to a file

e) go back to a) unless the "Stop" control has been clicked.

Note that hex values may be entered by prefixing an "&H" to the value

Measure

This facility allows the user to make either of two types of measurements on any one channel.  The two basic types of measurements are:

1) character: basically a UART and

2) count the number of transitions between the cursors

The data display method is used to determine the type of measurement as well as the radix for the UART type.

The allowable types are:  ASCII,  Hex,  Decimal,  Octal,  Binary,  + transitions and - negative transitions.

When selecting the UART measurement mode, you must set  the following parameters:

1) number of data bits (7, 8, 9)

2) number of stop bits (1, 1.5, 2)

3) the Mark state (1/0)

4) the location of the most significant bit - box not checked is MSB last

5) the baud rate - type in any value you want.  IMPORTANT: There must be at least two samples per bit!

The program will calculate both the time per bit and the time per character based on the above information.  It will search for the START bit and then for each of the data bits.  Once the leading edge of the START bit is located it will add one to the sample number in order to insure that the samples are within each of the bit times.

NOTE: if either of the transition options is selected, the UART parameters will be disabled

For both measurement types you must select:

1) the cursor memory number to use as the starting point and

2) the input channel

Activating the “OK” button will cause the Measurement window to disappear.  Then whenever the cursor position is changed, the new data value(s) will appear in the status box.

The status display is limited to approximately 135 characters.  If the number of characters to be displayed excedes this number the program will display as many full byte values as will fit and will turn the background color to pink as an overflow indication.

Sample INI File

; c:\la\la.ini

; The following 2 sections are necessary and are

; NOT available in the Configuration menu.

[PRINTER_PORT]

ADDRESS = &H278

HARDWARE = 1

[LA_MEMORY]

RAM_ADDR_BITS = 17

PRE_TRIG_BITS = 12

;=================================================

[LA_TIMEBASE]

SELECT_0 = 100ns

SELECT_1 = 200ns

SELECT_2 = 1us

SELECT_3 = 10us

SELECT_4 = 100us

SELECT_5 = 1ms

VALUE = 1

[LA_CHANNELS]

COUNT = 6

NAME_CH1 = A0

NAME_CH2 = A1

NAME_CH3 = A2

NAME_CH4 = A3

NAME_CH5 = TrigStart-H

NAME_CH6 = U602-19

NAME_CH7 =

NAME_CH8 =

[LA_TRIGGER]

TRIG_CH1 = X

TRIG_CH2 = X

TRIG_CH3 = X

TRIG_CH4 = 1

TRIG_CH5 = 1

TRIG_CH6 = X

TRIG_CH7 = X

TRIG_CH8 = X

[LA_CURSOR]

POSITION = 4096

MEM_1 = X

MEM_2 = X

MEM_3 = X

MEM_4 = X

SAMPLE_VALUE = CH8	;Ch8=MSB	OFF, CH1, CH8

SAMPLE_BASE = BIN	; DEC, HEX, OCT, BIN

[LA_WINDOW]

START_SAMPLE = 4070

END_SAMPLE = 4120

SLIDE = 90