QEXfiles: Baseband Quadrature Multi-Band Modulator

Braddon Van Slyke, ACØZJ

13111 W. 60th Ave., Arvada CO 80004

email:  blindpigsaloon@q.com

Baseband FM Modulation using Octave. See also basebandIQ.m

% Filename: basebandIQ.m

%

% Purpose: Generates FM or SSB IQ baseband data for use with the LTC5598

%          Quadrature Modulator Board.  The modulating source is a

%          previously recorded wav file, but it could be generated in

%          Octave/Matlab just as easily.

%

% B. Van Slyke AC0ZJ  1/3/2018

%

% Note - Octave users - The hilbert function requires you run

% 'pkg load signal' from the command line prior to running this script

clear

more off  % allows displaying variables during run

% load the modulating signal

[yfile, fs] = audioread('c:\temp\morseAudio.wav');

% data from audioread is in colums, one row.  Switch to one column for

% ease of use

y = yfile';

% need to subtract off any dc from input (audio) signal

y = y - mean(y);

% Normalize the amplitude of y such that it's same as a sine wave with

% peak-to-peak amplitude of 1.  WARNING - the audio contains portions of

% silence, so you can't use std - the more silence, the more it biases

% lower. Assuming a simple tone, just use the max of the signal to scale.

y = y  / max(y);

% Add zeros (silence) at the beginning of audio to allow 2m handheld

% squelch time.  Add to end as well to prevent abrupt ending.

y = [zeros(1, 4000) y zeros(1, 500)];

% optional test tone

% fs = 8000;

% t = 0 : 1/fs : 5 - 1/fs;

% y = sin(2*pi*t*1900);  %1.9 kHz 4.5 deviation => null at carrier

modgain = 2.4;

% Upsample the data.  If the baseband modulating frequency is to be 10

% kHz, then the final data rate should be at least 2x greater than that

% (Nyquist)

upsampleRate = 48000;

ys = resample(y, upsampleRate, fs);

fs = upsampleRate;

% Set to 1 (true) for FM modulation, 0 for SSB

fmMod = 0;

if fmMod == 1

t = 0: 1/fs : (length(ys) - 1) / fs;

% The baseband carrier frequency, that is, the signal to be modulated.

% If the LO, for example, is set to 145.0 MHz, then the transmitted

% signal will be at 145.010 MHz.  Negating Q will move the transmitted

% signal to 144.090 MHz.  The modulating signal must be band-limited

% to fc/2.

fc = 10000;

% This value was set by observing the RF output while transmitting a

% tone of 1.9 kHz.  By using the Bessel Null method, a deviation of

% 4.5 kHz can be set by adjusting the modulation gain so that the

% carrier is set to a minimum.

modulationGain = 2.4;

% Assuming a band limited modulation signal

% Integrate modulating signal up to current time using cumsum function

integralSumYs = cumsum(ys);

s = cos(2 * pi .* t * fc + modulationGain .* integralSumYs);

H = hilbert(s);

%  Here's an alternative to using the Hilbert transform

%  H = cos(2 * pi .* t .* fc + modulationGain .* integralSumYs);

%  H = H + i * -sin(2 * pi .* t .* fc + modulationGain .* integralSumYs);

else % assume usb

H = hilbert(ys);

% for LSB, multiply the Q signal by -1.

end

% Create array 'a' for output to audioplayer

a(1,:) = imag(H);

a(2,:) = real(H);

% This is sound card specific!!!

% Gain vs voltage out of sound card *speaker port*.  The "line level"

% output isn't high enough for the modulator board, as max 2 Vpp input is

% expected.

%

% gain   Vpp out FM

%  1.0     4.0v

%  0.8     3.2v

%  0.6     2.4v

%  0.4     1.6v

%  0.3     1.25v

%  0.2     0.83v

%  0.1     0.4

gain = 0.3;

a = gain * a;

% Display one channel of output for test/analysis

kk = 22000:23000;

plot(a(1,kk))

% Use audiodevinfo to get audio device info such as name of device

dev = audiodevinfo();

dev.output.ID

dev.output.Name

% *** You'll need to try different numbers to see what number matches to

% your specific sound card.

% Note - it seemed that "playblocking" didn't appear to work immediately

% with a new device after a usb sound device install, but a reboot fixed

% that.

outputDeviceID = 4;     % speakers USB sound device

numberOutputBits = 16;

player = audioplayer(a,fs,numberOutputBits,outputDeviceID);

% If you want to play more than once, use the loop

%for i = 1 : 3

% don't use 'play' as it will immediately return

%disp(i)

playblocking(player);

%end

% If you don't want to regenerate every time, here are save and load

% variable examples...

% save morseCode1 a

% load morseCode1 a